| For the attention of: | Conversion Reference # RDE 076 |
| --- | --- |
| Fax No. | Date: |

This information pack is to support an application for a Notifiable Alteration. The conversion Reference above must be quoted on form VTG10.

RENAULT 50 SERIES R1B56 (S56) REPLATED TO 6600 kg GVW

|  | Front Axles | Rear Axle | GVW | GTW |  |
| --- | --- | --- | --- | --- | --- |
| Standard Plating | 2150 Kg | 4000 kg | 5600 kg | 8100 kg |  |
| Required Plating | 2300 kg | 4900 kg | 6600 kg | 9100 kg |  |

Revised plating to be achieved by conversion of vehicle to condition recognized as standard Type Approved specification for R1B66 (S66) plated as above.

- Fit S66 rear springs (parabolic leaf) and hanger brackets

- Fit S66 rear axle (complete, including brakes)

- Fit S66 front disc brakes (complete, including discs, callipers and hubs)

- Fit new LSV data plate stamped with S66 data

- Reset LSV

To ensure the correct plating for the vehicle please submit a weight calculation with any request. If you do not have the facilities for this then contact the Renault Trucks Product Applications Department (01582 479131 or 479133) who will be pleased to assist you.

Plating now as shown above.  GTW as above for vehicles without drawbar brakes.  No type approval for drawbar brakes which will be covered by VTG10 to achieve a maximum as shown above.

If in doubt about any of the above procedure, you must refer to any RENAULT Trucks appointed dealer, who will be pleased to quote for this work.

Product Applications

RDE076

ADDENDUM FOR GOODS VEHICLE CENTRE

Type Approval No. 010A103T (S56), 010A104T (S66)

It is unlikely a case history will be needed for this alteration.

Front Axle

Springs unchanged

Twin calliper disc brake system fitted – same as S56 heavy duty system fitted to library specification chassis and buses.

Rear Axle

Revised spring details:

Part Number	0038320600

Number of leaves	3

Stack height	62 mm (S56 stack height is 57 mm)

Renault/Chrysler type AX8 drive axle and associated brakes fitted.